Digital Communications and Networks 11 (2025) 1751–1761

工

5

入

<

人

C

Q

Contents lists available at ScienceDirect

# Digital Communications and Networks

journal homepage: www.keaipublishing.com/dcan

| An eﬃcient downlink resource allocation scheme in power-limited GEO satellite systems ☆ Xin Tong * , Nanxi Li, Jiaxiang Liu, Shuo Peng China Telecom Research Institute, Beijing 102209, China |  |  | check for updates. |
| --- | --- | --- | --- |
| A R T I C L E I N F O |  | A B S T R A C T |  |
| Keywords: Power-limited GEO satellite Downlink resource allocation QoS Resource eﬃciency |  | The integration of Geostationary Earth Orbit (GEO) satellite constellations into Sixth Generation (6G) framework for cellular networks is essential to achieve global connectivity. Despite the major importance of this integration, current research often underestimates the limitations imposed by available satellite payload power, erroneously assuming a uniform maximum power density distribution across all communication beams. In this paper, we propose an Eﬃcient Downlink Resource Allocation scheme (EDRA) that accounts for transmitting power resource limitations, variable service quality demands, and a heterogeneous number of users. Our approach relies on the thorough analysis of real-world demographic data, allowing us to optimize the allocation of downlink power and time–frequency resources in a practical and eﬀective manner. Furthermore, we introduce an optimization model to maximize the total system revenue, using an iterative algorithm speciﬁcally designed to solve complex optimization problems. Numerical simulations demonstrated that the EDRA scheme improved the average network revenue by more than 66% relatively to standard methods, with performance gains increasingly large for an increasing diversity of service types, establishing the robustness and adaptability of the proposed EDRA scheme in the rapidly-evolving context of satellite-based communication systems. |  |

## 1.  Introduction

Satellite  networks,  particularly  those  operating  in  Geostationary Earth Orbit (GEO), can providing extensive connectivity and continu- ity of services in regions where the installation of terrestrial antennas is either geographically impractical or economically unviable  [1,2]. This allows satellite systems to fulﬁll a variety of requirements, oﬀering ro- bust support across diverse vertical industries and consumer electronics services [3]. This encompasses various sectors, including the automotive industry,  environmental  conservation,  military  communications,  and medical emergency response, with a particular emphasis on satellite- linked handheld terminals [4,5].

Considerable geographical disparities in satellite coverage and ter- restrial distribution of User Equipment (UE) or base stations, such as the “Next Generation Node B” (gNB) for the Fifth-Generation (5G) of cellu- lar network technology, strongly contribute to signal path loss [6–8].

This phenomenon is further exacerbated by a multitude of environ- mental  factors  that  adversely  aﬀect  signal  strength  and propagation, including atmospheric absorption, which attenuates signal strength; at- tenuation  caused  by  clouds  and  rain,  which  can  scatter  and  absorb

electromagnetic waves; and scintillation in the troposphere and iono- sphere, which causes ﬂuctuations in signal strength and phase  [9,10]. These factors degrade the signal transmission conditions, substantially compromising the eﬃciency and reliability of data transmission in satel- lite communication systems.

In satellite communication networks, there is a prevalent pursuit to bolster the quality of data transmission, primarily by increasing trans- mitting power and expanding antenna arrays  [11,12]. This approach aims at limiting signal attenuation and enhancing signal-to-noise ratio to improve the reliability and integrity of data transmission over long dis- tances. However, there are limitations to increasing transmitting power and antenna size. First, satellite launch capabilities, determined by phys- ical constraints on rocket technology and on the structural integrity of the satellite platform itself, limit the possible mass and size of satellite payloads  [13]. Additionally, the economic aspect must be considered: larger satellites with higher transmitting power typically incur substan- tially higher manufacturing, launch, and operational costs. In particular, compared with  Low  Earth  Orbit  (LEO)  satellites,  the  launch  of GEO satellites  is  considerably  more  complex,  requiring  enhanced  payload capabilities and more complex orbit transfer techniques. These factors

☆   Peer review under the responsibility of the Chongqing University of Posts and Telecommunications.

*  Corresponding author.

E-mail address: tongxin950729@163.com (X. Tong).

https://doi.org/10.1016/j.dcan.2025.05.007

Received 22 November 2024; Received in revised form 7 May 2025; Accepted 16 May 2025 Available online 19 May 2025

2352-8648/© 2025 Chongqing University of Posts and Telecommunications. Publishing services by Elsevier B.V. on behalf of KeAi Communications Co. Ltd. All rights are reserved, including those for text and data mining, AI training, and similar technologies. This is an open access article under the CC BY-NC-ND license (http://creativecommons.org/licenses/by-nc-nd/4.0/).

万方数据

X. Tong, N. Li, J. Liu et al.

jointly impose a practical limit on transmitting power and antenna size for satellite-borne devices. Consequently, strategic allocation of satellite downlink resources, especially the downlink transmitting power, has be- come an essential aspect of network optimization. It involves carefully balancing a large variety of user requirements of diﬀerent users, the geographical distribution of the user base, and speciﬁc constraints on service characteristics to maximize the overall network eﬃciency while ensuring that Quality of Service (QoS) meets end-user expectations.

With the increasing diversity of user proﬁles and QoS requirements [14], the complexity of satellite downlink resource allocation has in- creased  to  become  a  critical  determinant  of the  overall  communica- tion system capacity. As such, it necessitates sophisticated optimization strategies to ensure that the system can meet the increasingly large and diverse  demands  for  user  services while  maintaining  high  QoS  stan- dards.

### 1.1.  Related work

1.1.1. Assumptions on resource allocation in 3GPP

A considerable variety of standards has been developed for satellite communications, including Digital Video Broadcasting (DVB) standards [15] and several proprietary protocols. The current consensus within the industry is that the Non-Terrestrial Network (NTN) standard, devel- oped within the framework of the Third Generation Partnership Project (3GPP), may become the future dominant standard [16]. This is because the 3GPP, to leverage the unique characteristics and intrinsic design con- straints of satellite networks, has actively incorporated such networks into the 5G technological framework since the start of Release 15, the ﬁrst phase of the 5G standard release.

Subsequently, Release 17 and Release 18 have eﬀectively achieved the integration of satellite networks into the 5G New Radio (NR) stan- dard in 2024 [17]. This integration encompasses essential functionalities and enhancements critical for eﬀective operation of satellite networks within the 5G framework. Notably, the design paradigm in Release 17 and Release 18 assumes equal beam power subject to a maximum power density constraint, but does not consider power limitations of satellite payloads. In particular, standardization of power allocation remains un- addressed in Release 18 [18].

For the upcoming Release 19, the outlined research agenda focuses on handset terminals, aiming to address the limitations inherent to satel- lite payloads, speciﬁcally in relation with power availability that may prevent timely activation of all communication beams at the nominal Equivalent Isotropically Radiated Power  (EIRP) density. The primary research objective is to ensure complete service of all user terminals within the satellite footprint, by maximizing satellite throughput while keeping all satellite radio beams active even in the absence of traﬃc, thereby preventing any degradation in end-user QoS and allowing new users to join the network  [19]. Currently, the 3GPP has identiﬁed ef- ﬁcient power sharing and link-margin improvement through enhanced downlink resource allocation as potential strategies to mitigate the con- straints imposed by limited satellite power resources [20].

#### 1.1.2.  Research on resource allocation methods

Notable scientiﬁc analyses on optimizing downlink resource alloca- tion for satellite communication systems have recently been published.

Some studies concentrated on the design and optimization of objec- tives and algorithms for resource allocation. For example, an important study in [21] introduced a dynamic downlink resource allocation strat- egy in the context of a Low Earth Orbit (LEO) - High Altitude Platform (HAP) cognitive system. The described scheme successfully solves the imperfect estimation problem occurring before resource allocation by introducing a correction coeﬃcient that maximizes the HAP-to-ground downlink  capacity  without  compromising  the  LEO-to-ground  down- link QoS.  [22] introduced a bandwidth constraint management strat- egy predicated on the Non-Dominated Beam Coding (NDBC) method- ology. This method is designed to mitigate bandwidth overlap during

Digital Communications and Networks 11 (2025) 1751–1761

the evolutionary phase of algorithm population to achieve full band- width ﬂexibility, thereby optimizing the dynamic frequency resource management  solution  while  reducing  computational  complexity.  Ac- cording  to  the  authors,  integrating  the  non-dominated  beam  coding method into a metaheuristic framework simultaneously shortens the al- gorithm  execution  duration  and  improves resource  allocation.  In the work  denoted  by  [23],  a  novel  recursive  frequency  resource  alloca- tion algorithm was proposed, amalgamating Long-Short Term Memory (LSTM) networks with Proximal Policy Optimization (PPO). Speciﬁcally, the PPO algorithm is leveraged with an LSTM network to recursively for- mulate sub-actions about frequency resource allocation for each beam, which substantially condenses the action space. Moreover, the LSTM- driven recursive framework enables PPO to more eﬀectively allocate subsequent frequency resources by utilizing the information from the generated  sub-actions  as  antecedent knowledge,  thereby  diminishing the neural network’s complexity. Guo et al. proposed an eﬃcient multi- dimensional resource allocation framework for beam-hopping in LEO satellite networks in [24]. After formulating the resource optimization problem, the illumination pattern is characterized through a weighted greedy strategy, and then an enhanced genetic algorithm is applied for joint allocation of communication resources. This approach is particu- larly relevant when frequency resources are scarce and traﬃc demand is unevenly distributed. The study of  [25] reported on resource allo- cation algorithms for Integrated Satellite–Terrestrial Networks (ISTNs), in which a beam-hopping-based satellite system and terrestrial systems share  a  frequency band.  Because  frequency  allocation  eﬃciency  and user fairness are essential optimization metrics for practical systems, the authors formulated two resource allocation problems [25]: maximizing both the weighted sum capacity and the minimum capacity-to-demand ratio for integrated satellite–terrestrial networks, that they formalized as mixed-integer linear programming problems to derive optimal solutions.

[26] established a resource allocation model for Orthogonal Frequency Division Multiplexing  (OFDM) multi-beam high-throughput satellites, accounting for inter-beam interference, inter-sub carrier interference, and satellite downlink loss. The model’s objective is to ensure equitable resource distribution among satellite users, which is achieved through applying quantum genetic algorithms for resource allocation. [27] for- mulated a dynamic resource allocation problem that encompasses both user association and power allocation. The objective is to minimize the overall transmission latency, which is a complex issue that can be ef- fectively tackled using alternating optimization and successive convex approximation techniques. This approach underscores the importance of a holistic view of resource allocation, considering not only the al- location of power but also the strategic placement of users within the system.

In  addition  to  the  publications  mentioned  above,  several  studies have analyzed resource allocation in terms of bandwidth, power, and time.  For  example,  [28]  studied  the joint  downlink  power  and  time slot allocation problem to balance energy  consumption and total ca- pacity. Their study considered both the resource availability constraint and the combined coverage capacity of multiple satellites. They demon- strated the importance of characterizing the interconnections between these factors to achieve optimal system performance. In  [29], a new carrier and power allocation method using successive convex approxi- mation for ﬂexible broadband GEO satellite systems was proposed. Sim- ulation results demonstrated that this method outperforms benchmark schemes in terms of average per-beam demand matching, average con- sumed power, and bandwidth utilization. On this basis, [30] proposed a demand-aware radio resource allocation technique that minimizes the transmitting power and bandwidth of GEO satellites while ensuring that user demand is met. This demonstrated thorough understanding of the necessary balance between energy eﬃciency and user satisfaction. Fi- nally, [31] presented a multibeam GEO satellite resource optimization scheme. The author modeled the optimization problem as a Markov de- cision  process,  acknowledging  both  the  continuity  and  variability  of system resources. They proposed a deep reinforcement learning algo-

1752

rithm to jointly allocate power and bandwidth to satisfy beam traﬃc demand.

The studies mentioned above predominantly focused on optimizing power and bandwidth allocation or temporal power distribution, repre- senting not only a substantial progress in that ﬁeld of study, but also a motivation for further research into more sophisticated resource al- location strategies adapted to the constantly evolving requirements of satellite communication systems. For example, the study presented in [32] introduced both distributed and globally optimized algorithms for satellite-to-cell multibeam resource allocation. The purpose was to en- sure equitable distribution of time–frequency resources and beams to the cells, proportional to the number of connected users. However, the methodology in [32] primarily focused on assessing tradeoﬀs between average throughput and fairness without considering the inﬂuence of service diversity. In [33], an algorithm was proposed to simultaneously optimize downlink power, bandwidth, and time slots of a GEO satellite for enhanced energy eﬃciency. This bifurcated approach consisted in initially employing linear programming to allocate time slots to users, and then solving a non-convex optimization problem to determine the optimal power and bandwidth allocation for each user. Like [32], [33] assumed the homogeneity of service types among users, with resource eﬃciency as primary optimization goal. However, this assumption did not account for diﬀerences in QoS requirements between types of ser- vices  and their  impact  on  strategic resource  allocation. Additionally, [34] proposed a power and time slot allocation scheme relying on de- lay priority for multibeam satellite downlink. Its notable limitation was its exclusive focus on service delay constraints; consequently, the trans- ferred data volume was not included in the QoS metrics. Although these studies appropriately account for user distribution under diﬀerent beam conﬁgurations and for multidimensional resource allocation, they do not suﬃciently address the heterogeneity in service priorities, latency con- straints, and data volume requirements among user service types.

As development and standardization have started for the sixth gen- eration of cellular network technology (6G), the surge in “Internet of Things” (IoT) users and the increase in the diversity of services present an important opportunity to reﬁne current research paradigms. A more nuanced approach that accommodates a diversity of QoS demands and types of service is necessary to improve future satellite resource alloca- tion schemes.

1.2.  Motivation and paper organization

In the domain of GEO satellite communications, the limitation of downlink transmitting power poses a considerable problem, particularly in the current context of early development of 6G frameworks, when UEs require a broad range of services with increasingly direct and fast access to GEO satellites over vast geographical areas. The purpose of this study is to optimize downlink resource allocation by including fac- tors previously overlooked, such as service type prioritization, stringent low-latency requirements, and data volume variability. By addressing these gaps in the current scholarly discourse, this work aims to provide preliminary support for more robust and responsive satellite communi- cation systems.

The main contributions of the present paper are summarized as fol- lows:

•  We introduce an Eﬃcient Downlink Resource Allocation (EDRA) scheme in this paper, which is designed to optimize the allocation of downlink power and time-frequency resources under the con- straint of limited power. This scheme takes into account the power limitations, the QoS requirements for various applications, and the number of UEs, utilizing real-world population data for a compre- hensive analysis.

•  Within this framework, we formulate an optimization model aimed at maximizing the overall system revenue. Subsequently, we de- velop  an  iterative  optimization  algorithm  based  on  the  Karush-

### Table 1

Parameters used in this paper.

| Parameter | Meaning |
| --- | --- |
| 上D | Total transmitting power of satellite |
| Nu | Number of beams |
| 」 | Diameter per beam |
| 瘓 | Bandwidth per beam |
| 上‘ | Transmitting power per beam |
| 瘓卜瘓 | Bandwidth per RB |
| !D | Duration of each OFDM slot |
| N‘ | Number of active UEs in cell‘ |
| 驰 | Number of service types |
| !6曲6 | Time period |
| 阝 m吐几 | Maximum number of basic resources |
| ℎ“ | Channel fading coeﬃcient |
| 上2 | Power of the LOS signal |
| P2 | Power of the scattering signal |
| 上上酱 | Attenuation due to atmospheric gasses |
| 上上D | Attenuation due to either ionospheric or tropospheric scintillation |
| 上上 阝 | Rain and cloud attenuation |
| 「(“,D) | Antenna gain of UE towards satellite |
| 「(D,“) | Antenna gain of satellite towards UE |
| 「m(D) | Antenna gain of satellite towards UE |
| 酱(‘,“) | Downlink power-gain-to-noise ratio of UE“ in cell‘ |
| Ψ | Wavelength |
| Φ | Noise power |
| ‘ | Distance between satellite and UE |
| 「 | Angle between UE and satellite |
| 4 | Distance between satellite and beam center point |
| P(‘,“) | Upper-bounded throughput for any UE |
| U“阝 | Number of basic resources allocated to UE“ |
| 上 (“) cov | Reception success probability of UE“ |
| / | Downlink power-gain-to-noise ratio threshold for successfully receiving data |
| 卜(‘,“) | Revenue of UE“ in cell‘ |
| Ω | Sum of the revenue of all UEs |
| "P阝匕(“) | Service priority coeﬃcient of UE“ |
| !P阝匕(“) | Service delay constraints of UE“ |
| P阝匕 N (“) | Data volume of UE“ |
| cov,min − 上 | Reception success probability constraint |
| Ω | Average revenue |
| 」P阝匕(“) | Default priority level of UE“ |

Kuhn-Tucker (KKT) conditions to address the optimization problem presented eﬀectively.

•  To assess the performance of the EDRA scheme, we conduct ex- tensive numerical simulations. These simulations demonstrate the superior resource utilization capabilities of EDRA. The results in- dicate that EDRA can signiﬁcantly enhance the network’s average revenue by over 66% when compared to existing methodologies. Furthermore, as the diversity of service types increases, the perfor- mance advantage of EDRA becomes even more pronounced.

The rest ofthe paper is organized as follows: ﬁrst, the general system model is presented in Section 2. Section 3 describes the EDRA optimiza- tion problem and the proposed iterative optimization algorithm, both evaluated through numerical simulation analyzed in Section 4. Finally, Section  5  summarizes  the  study  and  presents  perspectives  for  future work. A list of the main parameters used in this study is given in Ta- ble 1.

## 2.  System model

### 2.1.  Network architecture

In this study, in compliance with the general framework outlined in the 3GPP Release 18 documentation, we investigate an NTN downlink scenario with a GEO satellite deployed in a transparent relay conﬁgura- tion, as illustrated in Fig. 1.

In this scenario, the role of base station is attributed to a single gNB responsible  for  resource  allocation, while  the  satellite  functions  as  a

1753

# Fig. 1. Network architecture, in which the GEO satellite serves UEs on the ground through a direct access link at the S band.

# Fig. 2. Frame architecture when subcarrier spacing is 15 kHz.

passive relay responsible only for signal ampliﬁcation and subsequent transmission via S-band communication links. The total satellite trans- mitting power, noted 上D , is unevenly distributed across Np beams. Each beam is allocated a speciﬁc geographical service area of diameter 」, analogous to a cell in cellular networks, and operates with a bandwidth瘓 and transmitting power 上‘ .

The system uses the Orthogonal Frequency-Division Multiple Access (OFDMA) scheme deﬁned by the 3GPP for 5G New Radio NTNs  [17]. In this system, the fundamental unit for frequency domain resource al- location is the Resource Block (RB). A RB encompasses a sequence of 12 contiguous subcarriers, occupying a bandwidth 瘓卜瘓 of 180 kHz. As- suming a subcarrier spacing of 15 kHz as reference scenario, 25 RBs can be accommodated within a 5-MHz bandwidth. Therefore, we deﬁne 15 kHz as subcarrier spacing in our analysis and use RBs as fundamental unit for frequency-domain resource allocation. In the time domain, the NTN frame duration is 10 ms; thus, with a 15-kHz subcarrier spacing, each frame comprises 10 slots. Taking a subcarrier spacing of 15 kHz as an example, the frame structure based on OFDM is shown in Fig. 2. In this paper, we set the time domain allocation unit as a slot, and each OFDM slot occupies !D  seconds in the time domain. Consequently, we deﬁne a basic time–frequency resource unit of dimension 1 RB in the fre- quency domain and 1 slot in the time domain to streamline subsequent analyses.

The UEs are aggregated into Np cells, and the number of active UEs in the cell‘ ∈ Np  is N‘, which is related to the local population lev- els. We model the locations of UEs within a given cell as independent and homogeneous Poisson point processes with a mean of“‘  [32]. Fur- thermore, we assume that there are 驰-types of services in the system, each with its unique QoS requirements [35]. Each UE“ ∈ N‘ is assigned

U“∫ basic resources within each time period !‘曲‘ and 瘓 frequency band according to the optimization problem below, where the maximum re- source allocation number is equal to

### 2.2.  Channel model

In contrast to terrestrial communication system, only outdoor condi- tions are considered for satellite operations in 3GPP, since performance requirements are not expected to be met with the available link bud- get for indoor communications. In this paper, we exclusively consider outdoor UEs, while all UEs are assumed to directly link to the GEO satel- lite. The signal transmission from the GEO satellite to UEs propagates over  the  air  directly without  any  obstruction  [36,37].  Consequently, in satellite communication systems, the Line of Sight (LoS) signal typ- ically represents the most potent signal  [36]. Hence, it is reasonable to consider the microwave channel for downlinks as a Rician fading channel with additive Gaussian noise. In addition, by referring to the characterization of satellite channels in ITU-R P.618 [38], we incorpo- rate atmospheric eﬀects such as gas attenuation, scintillation, and rain and cloud attenuation.

Therefore, the channel fading coeﬃcient““ for UE“ ∈ N‘ in the cell ‘ can be modeled as a circularly symmetric complex Gaussian random variable [10], and the distribution of Ⅱ““Ⅱ2 is given by non-central chi-	squares distribution probability density function

(2)

where 上2 = μ 12 + μ22  is the power of the LoS signal, P2  is the power of the scattering signal, and !0 (∙) is the zeroth order modiﬁed Bessel function of the ﬁrst kind, 上上为  is the attenuation due to atmospheric gasses, 上上D is the attenuation due to either ionospheric or tropospheric scintillation, and 上上∫ is the rain and cloud attenuation [38].

### 2.3.  Upper-bounded throughput and reception success probability for any UE

Let「(“,D)  and「(D,“) be the antenna gain of UE“ towards the beam of the GEO satellite, and vice versa, respectively. The downlink power- gain-to-noise ratio 为(‘,“)  of UE“, which can be denoted by

1754

where (匕,“) refers to the UE“ in cell 匕, Ψ is the wavelength, Φ is noise power, and‘ is the distance between the GEO satellite and the UE [28].

The antenna gain of the satellite to UE 「(D,“)  is related to the UE’s position in the beam, and it decreases as the angle between UE and the satellite V increases [39]. Considering that the eﬀective diameter of each beam in the GEO satellite is relatively large, we suppose that「(D,“) decreases linearly as V increases. We assume that the position of each UE within the beam follows a two-dimensional Poisson distribution, so the average distance between UE and the center point of the beam 4, as well as the mean angle between it and the center point of the beam V are both“匕 . Therefore, for UE with a distance of 4 from the center point of the beam, its「(D,“)  can be expressed as

where Δ=34匕. Moreover, the distance between the GEO satellite and the UE can be calculated by‘ = 4∕sin V匕 .

Then, the throughput for any UE is upper-bounded by

where U“阝 is the number of basic resources allocated to UE“.

Moreover,  the  diverse  QoS  requirements  associated  with  distinct	types of services necessitate varying levels of reception success prob-		ability to ensure optimal performance and reliability. Consequently, the	probability  of successful  reception  should  be  incorporated  as  a  con-		straint within  the  resource  allocation  process.  The  fulﬁllment  of the Signal-to-Noise Ratio (SNR) constraint is a signiﬁcant determinant of	the probability of successful reception. Thus, in this paper, we adopt	the probability of satisfying the SNR constraint as a surrogate measure	for the data reception success probability. Based on the above analysis, we can further obtain that the data reception success probability of UE “, which is

上cov (“)  = P[上匕 (匕,“) ≥ /]

where / is the downlink-power-gain-to-noise ratio threshold for success- fully receiving data. We will use the reception success probability as one of the constraints for resource allocation in the following.

## 3.  The EDRA scheme: design and optimization

Limited downlink transmitting power is a serious problem for GEO satellite communications, particularly in the early development stages of the 6G framework. Although the most recent algorithms appropriately account for the distribution of users under diﬀerent beam conﬁgurations and for multidimensional resource allocation, they do not suﬃciently address the heterogeneity in service priorities, latency constraints, and data volume requirements among user service types. For these reasons, we propose the EDRA scheme that considers power limitations to opti- mize downlink resource allocation. In this section, we detail the EDRA operational process and optimization objectives; we also design an iter- ative optimization algorithm to solve the deﬁned optimization problem.

3.1. Algorithm workﬂow

Designing  an  algorithm  with  minimal  deviations  from  the  estab- lished standards demonstrably increases its chances of later adoption for practical applications. Therefore, we have developed the EDRA al- gorithm in accordance with 3GPP speciﬁcations [20]. Within the 3GPP NTN framework, the data scheduler, an integral component of the gNB, operates at the Medium Access Control (MAC) layer and is tasked with

# Fig. 3. The main process of EDRA.

managing resource allocation for both uplink and downlink data trans- missions to and from all UEs within its coverage area [40]. For eﬃcient scheduling, the gNB must collect data from each UE, including the status of its data buﬀer and its QoS requirements [41]. The original methodol- ogy introduced in the EDRA scheme deviates from standards by focusing on the gNB calculation of the content popularity probability, noted 上匕and  U“阝 , to optimize the system downlink capacity in line with pre- deﬁned  optimization  goals.  Following  these  computations,  downlink resources are allocated strategically to maximize network eﬃciency and performance. The EDRA algorithm workﬂow is illustrated in Fig. 3.

The critical processing steps are as follows:

•  A UE initiates the Scheduling Request (SR) process when it requires data transmission. During this process, the UE sends a 1-bit SR to the gNB on its designated Physical Uplink Control Channel (PUCCH) SR resources. This action signals to the gNB that data is ready for transmission.

•  Upon detecting the SR, the gNB assigns resources that allow for the UE to transmit a buﬀer status report (BSR). The UE then transmits a MAC protocol data unit that includes the BSR describing the al- located gNB resources and that also contains details about service priority, delay constraints, and default maximum data burst volume that the user intends to transmit.

•  The gNB then reﬁnes resource allocation on the basis of the re- ceived BSR, using the optimization model detailed in Section 3.2. By leveraging the user service information and in conjunction with the optimization model, the gNB determines the resource alloca- tion strategy. It communicates the power allocation outcomes to each beam of the satellite through the feeder link. Furthermore, the gNB informs the UE of the time-frequency resource allocation via the Time Domain Resource Assignment and Frequency Domain Resource Assignment ﬁelds within the Downlink Control Informa- tion (DCI) transmitted over the Physical Downlink Control Channel (PDCCH) within the service link.

1755

3.2.  Optimization problem

We formulate an optimization problem to allocate the available satel- lite power to cells and cell time–frequency resources to UEs. In 3GPP technical speciﬁcations document TS  23.501  [19],  QoS requirements are explicitly deﬁned for each type of service, with details on the most common services, default priority levels, latency constraints, and default data volumes.  First, when  the  resource  allocation mechanism  priori- tizes services with higher priority levels, the corresponding UE clearly achieves a higher revenue, in direct proportion. Second, services that require larger data volumes necessitate more resource allocation, which increases the system overhead necessary to fulﬁll the resulting trans- mission requirements. Consequently, the revenue decreases as the data volume increases. Third, delay constraints for service transmission must also be satisﬁed. Thus, revenue increases when the diﬀerence between the speciﬁed latency constraint and the actual transmission delay in- creases, because  a larger  diﬀerence  indicates more  eﬃcient resource utilization and better compliance with service requirements.

Considering these factors, we express 卜(‘,“), the revenue of a UE, as in Eq. (7). The system revenue Ω is then deﬁned as the sum of all UE revenues within the system. The objective of the optimization model is to maximize system revenue.

卜(‘,“) = "p/‘                                                                       (7)

where "p/‘(“) is the service priority coeﬃcient of UE“, !p/‘(“) is the ser- vice delay constraints of UE“, Np/‘(“) is the data volume of UE“.

We aim to determine the values of 上‘ and U“/ , the power allocation of the GEO satellite to the cells and time-frequency resources to the UEs in one cell, so that the resources are eﬃciently distributed across the cells in the area.

The following constraints are deﬁned for the allocation. First, one satellite can allocate up to 上D  power to each cell, and the sum of the power allocated to all beams cannot exceed 上D . Second, the power given to each beam cannot be too small, at least meeting the reception suc- cess probability requirements. Third, the number of OFDM resources occupied by each UE cannot exceed /max, while the sum of resources allocated to all UEs in the same cell cannot exceed  /max . With these constraints in place, we formulate the optimization problem shown in Eq. (8), and the constraints are as follows.

max Ω

上‘,U“/

(8)

上cov(“) ≥ 上cov,min

where Ω is shown in Eq. (9).

3.3.  Iterative optimization algorithm

The formulated optimization problem is non-linear. Therefore, we ﬁrst separate Eq. (8) into two optimization subproblems, and then de- sign an iterative algorithm to reduce the computational complexity of the optimization problem and obtain a manageable solution.

3.3.1.  Subproblem of optimizing U“/

Deﬁning  matrices  N = {U“/l1 ≤‘ ≤ Np , 1 ≤“ ≤ U‘,max },  where U‘,max is the maximum number of UEs in all cells. If the number of UEs N‘ in cell‘ is less than U‘,max, U“/ =0 when N‘ <“ ≤ U‘,max.

When we focus on the optimization of U“/ , the optimization problem can be rewritten as follows by ﬁxing 上‘ .

D.‘. 0 ≤ U“/

(10)

U“/ ≤ /m吐几

U“/ ≤ /m吐几where

A1(‘,“) = "p/‘(“)!p/‘(“)

A2(‘,“) = "p/‘(“)Np/‘(“)/m吐几

A3(‘,“) = 瘓log2 [1 + 上‘「(“,D)「(D,“)EⅡ““Ⅱ2 Ψ∕(16兀2‘2 Φ)]

To solve Eq. (10), we initially investigate a relaxed problem where the number of assigned frames can take real values, i.e., within a contin- uous interval. The objective function expressed in Eq. (10) is a convex function that can be solved using the KKT condition. Additionally, meet- ing the third constraint implies that the second constraint is also met. Therefore, the optimal number of assigned resources  U“/ ∗ can be de- rived with the Lagrangian function deﬁned in Eq. (15) (relocated to the Appendix to maintain manuscript clarity).

By searching the feasible solutions located most closely to U“/ ∗ , the optimal solution of Equ. (15) N∇ can be expressed as Eq. (11).

N∇ ={arg min(D“mlU“/∈{0,/max,「U“/ ∗l , lU“/ ∗」})l1 ≤‘ ≤ Np ,

1 ≤“ ≤ U‘,max }                                       (11)

3.3.2.  Subproblem of optimizing 上‘

Deﬁning matrices P ={上‘l1 ≤‘ ≤ Np }. When we focus on the opti- mization of 上‘, the optimization problem can be rewritten as follows by ﬁxing U“/ .

D.‘. 0 ≤ 上‘ ≤ 上D

(12)

P[A6(‘,“)上‘ ≥ /] ≥ 上cov,min

where

A4(‘,“) = U“/ 瘓

A5(‘,“) =「(“,D)「(D,“)EⅡ““Ⅱ2 Ψ∕[(4兀‘)2 Φ]

A6(‘,“) =「(“,D)(「(D,“) −Δ)ΨJ Ⅱ““Ⅱ2 (“)∕[(4兀‘)2 Φ]

It is easy to show that the objective function displayed in Eq. (12) is a convex function, which can also be solved using the KKT condition. By applying the Lagrangian function detailed in Eq. (16), we can deter- mine the optimal beam transmitting power 上‘ ∗ . For the convenience of readers, Eq. (16) is placed in the Appendix.

The  optimal  solution  of  Eq.  (16)  P∇ can  be  expressed  as  P∇ = {上‘∗ l1 ≤‘ ≤ Np }.

3.3.3.  Joint optimization algorithm

As demonstrated by Algorithm 1, Eq. (8) can be solved by iteratively updating the solutions of two optimization subproblems. Speciﬁcally, in

1756

each iteration, N∇ and P∇ values are updated by solving Eqs. (15) and (16), respectively.

Algorithm 1 A Joint Optimization Algorithm of Eq. (8).

Input: the maximum iteration times 上max, the system revenue threshold D“m6“, N∇ , and P∇ Output: optimal N∇ and P∇ , system revenue Ω 1: initial P∇ ={上D ∕NP ¦1 ≤‘ ≤ NP }

2: initial N∇ ={/m吐几 ∕u‘ ¦1 ≤‘ ≤ NP , 1 ≤“ ≤ N‘ };

3: repeat

4: Update N∇ based on Eq. (15);

5: Update P∇ based on Eq. (16);

6: Calculate the value D“m = D“m1 +D“m2 by substituting the update results N∇ and P∇ ;

7: until (‘>上max or D“m‘−1 − D“m‘ ≤ D“m6“);

8: Calculate system revenue Ω= D“m.

The simultaneous iterative search of optimal solutions to both sub- problems ensures that the total system revenue Ω, expressed in Eq. (8), monotonously increases when iteration index‘ is incremented. There- fore, the optimization algorithm gradually converges toward a station- ary optimal solution, with a ﬁnite number of iterations [42].

Concurrently, we manage algorithmic complexity. Nominally, con- vex optimization problems are characterized by a polynomial-time com- putational complexity. By decomposing the optimization function into two independent convex subproblems and imposing constraints on the number of iterations, we ensure that the overall complexity of the pro- posed  algorithm  remains polynomial.  In  future  research,  algorithmic complexity may be further reduced by applying distributed optimiza- tion techniques [9] and AI-based methods [3].

## 4.  Numerical simulation for algorithm performance assessment

In  this  section,  we  analyze  representative  simulations  conducted with  MATLAB  R2021a  to  evaluate  the  performance  of the  proposed EDRA scheme and compare our results with the previous studies of [32] and  [34]. Both studies analyzed the joint allocation of time slots and power resources, but with distinct allocation principles. In particular, although the resource allocation algorithm proposed in [32] is designed for LEO satellites, its core algorithm can also be applied to resource al- location for GEO satellites. The allocated resource types are consistent between the two studies, but distinct from those in our EDRA scheme; however, the objective functions implemented in their resource alloca- tion models exhibited notable diﬀerences. The main characteristics of the allocation schemes in [32] and [34] are outlined hereafter.

•  Scheme  1  in  [32]:  a  fair  allocation  scheme, which  allocates  the power based on the number of UEs in each cell, allocates the num- ber of OFDM resources evenly within a cell.

•  Scheme 2 in  [34]: a joint power and timeslot allocation scheme, which allocates the power and number of OFDM resources based on the delay priority of UEs.

For comparison purposes, we evaluate the network’s performance in terms of the average revenue of the network, which is deﬁned as follows.

4.1.  Simulation settings

For our simulation, we selected ﬁve markedly distinct services from the 3GPP TS 23.501 document and, accordingly, set the number of ser- vice types to  驰 = 5. The corresponding default priority levels, delay budgets, and data volumes are listed in Table 2. The tabulated latency

represents the eﬀective timing constraint, after subtracting the propa- gation delay associated with the space-to-ground communication link.

The priority coeﬃcient variation is inversely proportional to that of the default priority level according to the following equation: assigning the lowest coeﬃcient value of 1 to a default priority level of 80 (Table 2); higher default priority levels (smaller numbers) of a service correspond to higher values of the priority coeﬃcient.

where 」p/‘ is the default priority level.

For practical evaluation of the algorithm performance relatively to the number of UEs and the required QoS, we designed three simulation scenarios.

•  Simulation Scenario 1: In this scenario, there are NP cells, and the number of UEs in each cell corresponds to the population distribu- tion. Each cell exclusively oﬀers a single type of service, meaning that all UEs within a cell have the same service type. The service type of each cell is randomly selected from the aforementioned 驰types of services, with selection probabilities of 10%, 22.5%, 35%, 22.5% and 10%.

•  Simulation Scenario 2: In this scenario, there are NP cells, and the number of UEs in each cell corresponds to the population distribu- tion. In each cell, all 驰 types of services are available, with each service type representing a distribution of 10%, 22.5%, 35%, 22.5% and 10%.

•  Simulation Scenario 3: In this scenario, there are NP cells, and the number of UEs in each cell corresponds to the population distribu- tion. The service type of each UE, and the number of service types are all randomly conﬁgured in each cell.

The other simulation parameters are shown in Table 3.

### 4.2.  Simulation results

At each iteration, the number of active UEs and their service types are randomly generated, in accordance with the population distribution. This process is repeated 100 times for each scenario to provide statistical robustness. To account for the stochastic generation of data within our simulation framework, results for the mean revenue distribution across the network are illustrated with box plots.

4.2.1. Average revenue in Scenario 1

−

Results for the average network revenue Ω in Simulation Scenario 1 for EDRA, Scheme 1  [32], and Scheme 2  [34] are compared in Fig. 4. Fig. 4(a) illustrates the statistical distribution of outcomes from the 100 simulations, while Fig. 4(b) focuses on the results of the fourth simula- tion.

Fig. 4(a) clearly demonstrates the consistently higher performance, −

as measured by  Ω, of Scheme  1 relatively to Scheme  2 under Simu- lation Scenario  1, particularly when the number of beams is limited. This can be primarily attributed to the scenario: in Simulation Scenario 1, all users within a single cell are engaged in identical services, re- sulting in equal time–frequency resource allocation in Schemes 1 and 2. However, despite the equal resource allocation, the power distribu- tion method markedly diﬀer between the two schemes. For scenarios in which a cell contains a large number of users with low delay priority, Scheme 1 allocates power preferentially to that cell, whereas Scheme 2 reduces the transmitting power of the beam associated with that cell, ul- timately limiting the average total revenue. This discrepancy increases under limited beam availability conditions, i.e., for a reduced number of cells. The EDRA strategy eﬀectively circumvents the limitations of both Scheme 1, which does not distinguish user service types, and Scheme 2,

1757

##### Table 2

QoS requirements.

| Default priority level | Priority coeﬃcient | Delay budget | Default maximum data burst volume | Example services |
| --- | --- | --- | --- | --- |
| 5 | 16 | 60 ms | 500 bits | Mission critical delay sensitive signaling (e.g., MC-PTT signaling) |
| 19 | 4.21 | 15 ms | 255 bits | Discrete automation, V2X messages |
| 30 | 2.67 | 50 ms | 1125 bits | Process automation monitoring |
| 70 | 1.14 | 100 ms | 1354 bits | Voice, Video (live streaming), AI/ML model download |
| 80 | 1 | 300 ms | 1700 bits | Video (buﬀered streaming) |

#### Table 3

Simulation Parameters value.

| Parameter | Meaning | Value |
| --- | --- | --- |
| Np | Beam number | 2-50 |
|  | Bandwidth per beam | 30 MHz |
| Ψ | Wavelength | 10 cm |
| 卜 | Bandwidth per RB | 180 KHz |
| Φ | Noise power | -122.2 dB |
| 上D | Total transmitting power | 200 W [34] |
| 「(“,D) | Antenna gain of UE towards satellite | 2 dB |
| 「m(D) | Antenna gain of satellite towards UE | 50 dB |
| !‘P‘ | Time period | 1 s |
| !D | Duration of each OFDM slot | 1 ms |
| cov,min 上 | reception success probability constraint | 90% |

which  does  not  consider  the  number  and  distribution  of users.  Con- −

sequently, the proposed EDRA scheme achieves a minimum  Ω value consistently exceeding those of Schemes 1 and 2, independently of the number of beams.

Furthermore,  as  illustrated  in Fig.  4(b), for  a  case with  only two beams, the number of UEs and their service types within each cell are consistent in the fourth simulation for the three algorithms, which thus achieve identical performance under these speciﬁc conditions. Because of their consistency with the statistical analysis results, the simulation results displayed in Fig. 4(b) are considered representative and corrob- orate the theoretical predictions.

Incidentally,  the  stochastic  nature  of these  simulations  prevented us from comparing performance disparities relatively to the number of beams for a speciﬁc allocation scheme. For example, in Fig. 4(a), the higher average EDRA revenue for Np = 50 than for Np = 30 does not ensure better performance when the number of beams is set to 30 in this scenario.

4.2.2. Average revenue in Scenario 2 and 3

−

The average revenue of the network Ω for EDRA, Scheme 2  [32], and Scheme 2 [34] in Simulation Scenario 1 and Simulation Scenario 3 are depicted in Fig. 5 and Fig. 6.

Figs.  5  and  6  further  illustrate  the  clearly higher performance  of the EDRA scheme. In Simulation Scenarios 2 and 3, user service types are heterogeneous within individual cells. Notably, in Simulation Sce- nario 2, a single cell is tasked with accommodating all service types. Consequently, Scheme  1, which considers equal distribution of time– frequency resources and transmitting power among users, cannot suit- ably account for this variability. Additionally, although Scheme 2 incor- porates user service delay priority into its resource allocation strategy, it does not account for service delay constraints and data volume vari- ability. Consequently, its performance is poorer than that of the EDRA scheme.

A comparison of Figs. 4(a), 5, and 6 indicates that, for a given number of beams,  the  proportion  of simulations  in  which  Scheme  2  outper- forms Scheme  1 ﬁrst increases and then decreases when the number and type of services increase. Speciﬁcally, this proportion ﬁrst increases substantially, from approximately 35% to 97% of the total number of simulations, and then decreases slightly to approximately 93%. We at- tribute these variations to the fact that Scheme 1 does not consider QoS diﬀerences, whereas Scheme 2 prioritizes UE latency, thereby achieving

(a)  statistics results of average revenue in simulation scenario 1

(b) Average revenue of the fourth simulation in simulation scenario 1

### Fig. 4. Average revenue in simulation scenario 1, which makes it evident that the EDRA exhibits the best performance.

better performance in scenarios with higher service type variability. Ad- ditionally, compared with Simulation Scenario 2, Simulation Scenario 3 and Simulation Scenario 1 represent more similar situations, with a sin- gle UE service type within each cell, while Schemes 1 and 2 both assign an equal number of OFDM resources  U“/  to all UEs within each cell. Considering the variation in UE numbers and its prioritization of service latency, Scheme 1 may achieve better performance than Scheme 2. Con- sequently, in Simulation Scenario 3, the proportion of instances where

1758

# Fig. 5. Statistics results of average revenue in Simulation Scenario 2.

# Fig. 6. Statistics results of average revenue in Simulation Scenario 3.

Scheme 2 demonstrates better performance than Scheme 1 maybe lower than in Simulation Scenario 2.

-

From  further  comparison  of  the  same  ﬁgures,  the   Ω  increase achieved  with  EDRA  relatively  to  Scheme  1  is  66%,  8.48,  and  5.93 for Simulation Scenarios 1, 2, and 3, respectively. A possible reason for this uneven performance improvement may be the diversity of services in Simulation Scenario 2, yielding poorest performance when Scheme 1 is  applied  to  distribute  OFDM  frames  evenly  among  UEs.  In  Simula- tion Scenario 3, the service type is identical for all UEs within a cell, reducing Scheme 1 performance and, thereby, resulting in a more mod- erate performance improvement with the EDRA scheme. Compared with

-

Scheme 2, the Ω increase achieved with EDRA is 1.02, 1.11, 1.79 for Simulation Scenarios 1, 2, and 3, respectively, representing a gradually stronger performance  improvement. We  attribute this to  comprehen- sive consideration, in the EDRA scheme, of service delay priority, delay constraints, and data volume requirements. With increasingly random- ized service types, the EDRA performance advantage becomes notably larger.

Our simulation results established that the EDRA scheme, overall, improves average network revenue by more than 66%. The simulation

results validate the advantages of EDRA. In the aforementioned sim- ulation, we set the number of service types to ﬁve and analyzed the performance of EDRA under three diﬀerent simulation scenarios. When the network service proﬁle was reduced to a single type, the EDRA per- formance was comparable with that of Schemes 1 and 2, as illustrated in Fig. 4(b). This observation suggests that, under homogeneous ser- vice conditions, the EDRA scheme does not improve on the standard allocation  algorithms.  However, when network  service heterogeneity increased—speciﬁcally through the expansion of service types, the intro- duction of more stochastic user distributions, and the randomization of service types for each user—the intrinsic adaptability and optimization capabilities of the EDRA scheme yielded a clearer performance improve- ment.

## 5.  Conclusion

Limited downlink transmitting power is a strong constraint for GEO satellite communications, particularly with the development of the 6G framework. Although standard allocation methods consider user distri- bution under a variety of satellite beam conﬁgurations and allow for multidimensional resource allocation, they are not suited to process an increasing variety of user services with considerable discrepancies in service priorities, latency constraints, and data volume demands.

In  this  study,  we  introduced  an  eﬃcient  downlink  resource  allo- cation scheme, speciﬁcally designed for GEO satellites on the basis of empirical population data to alleviate the limitations imposed by power constraints, diverse QoS demands across user services, and discrepancies in user equipment distribution. We established an optimization model to maximize the overall system revenue. For this purpose, we developed an iterative optimization algorithm that applied the Karush-Kuhn-Tucker conditions to eﬃciently solve the optimization challenges presented.

Our  numerical  simulation  demonstrated  that  the  EDRA  scheme markedly outperforms existing approaches, with tentative improvement of the network average revenue by 66% or more. To our knowledge, this is the ﬁrst study in integrating considerations of downlink power limi- tations, the real-world geographical UE distribution, and practical QoS requirements for UEs within the scope of downlink resource allocation.

Moreover, EDRA can also be adapted to LEO satellites and to net- work architectures integrating both GEO and LEO satellites. However, in such cases, additional work is required for intersatellite and interorbit coordination between satellites in diﬀerent orbits. In future research, we intend to further optimize EDRA for application to increasingly diverse satellite communication scenarios and to investigate possible algorithm complexity reduction, necessary to adapt the proposed EDRA scheme to the emerging regenerative satellite payload architecture.

CRediT authorship contribution statement

Xin  Tong:  Writing  –  original  draft,  Methodology,  Investigation, Funding acquisition, Formal analysis, Data curation, Conceptualization. Nanxi Li: Writing – review & editing, Software, Resources, Project ad- ministration, Methodology. Jiaxiang Liu: Writing – original draft, Vi- sualization, Validation,  Supervision.  Shuo Peng: Writing – review  & editing, Resources, Project administration, Investigation, Formal anal- ysis.

Declaration of competing interest

The authors declared no potential conﬂicts of interest with respect to the research, author-ship, and/or publication of this article.

Acknowledgements

This work was supported by Young Elite Scientists Sponsorship Pro- gram by CAST (No. 2022QNRC001).

1759

Appendix A

Eq. (15) and Eq. (16) are as follows.

I         ‘=1“=1 A3(‘,“)U“/             1

I( 上N  = NPΣ ‘    A2(‘,“)   - A  (‘,“)

I

I      NP  N‘

I  - Σ  Σ“1(‘,“)U“/

I    ‘=1“=1

I

I     NP                 N‘

I   ‘=1      “=1

I  + Σ“2(‘)( Σ U“/ - /m吐几)

I

I   匕匕上U“N/∗ I U“/ =U“/ ∗ =0

亻                                                                                                 (15)

I“1(‘,“),“2(‘) ≥ 0

I

I

I“1(‘,“)U“/ ∗ =0

I

I          N

I“2(‘)( Σ‘ U“/ ∗ - /max)=0 I        “=1

I         ∗

I -U“/   ≤ 0

I

I  N

lI“1 U“/ ∗ - /max ≤ 0

where“1(‘,“) and“2(‘) represent the constraint coeﬃcients correspond- ing to the Lagrange function.

I 上上 = Σ  Σ A  (‘,“)log [1+A  (‘,“)上 ] - A1

(          NP  N‘               A2(‘,“)                 (‘,“)

I         ‘=1“=1  4       2     5    ‘

I

I     NP                                 NP

I  -Σ“3(‘)上‘+“4( Σ 上‘ - 上D)

I   ‘=1              ‘=1

I

I     NP

I   ‘=1

I  + Σ“5(‘)(上cov,min - [A6(‘,“)上‘/])

I

I   匕上匕上‘ 上‘=上‘∗ =0

I

I“3(‘),“4,“5(‘) ≥ 0

I

亻                                                                                                 (16)

I“3(‘)上‘∗ =0 I

I        NP

I     ‘=1

I“4( Σ 上‘∗ - 上D )=0

I“5(‘)(上cov,min - [A6(‘,“)上‘∗ ≥ /]) = 0

I

I       ∗

I -上‘   ≤ 0

I

I

I  NP

I‘=1

I   Σ 上‘∗ - 上D ≤ 0

l 上cov,min - [A6(‘,“)上‘∗ ≥ /] ≤ 0

where“3(‘,“),“4(‘,“) and“5(‘) represent the constraint coeﬃcients cor- responding to the Lagrange function.

References

[1]  J. Wigard, E. Juan, J. Stanczak, M. Lauridsen, A. Marcone, S. Hoppe, A. Ahmadzadeh,

A. Masri, D.-H. Tran, Ubiquitous 6G service through non-terrestrial networks, IEEE Wirel. Commun. 30 (6) (2023) 12–18.

[2]  M. Al-Hawawreh, M.S. Hossain, Federated learning-assisted distributed intrusion de- tection using mesh  satellite nets  for  autonomous vehicle protection,  IEEE Trans. Consum. Electron. 70 (1) (2024) 854–862.

[3]  W. Wei, L. Fu, H. Gu, X. Lu, L. Liu, S. Mumtaz, Iris: toward intelligent reliable rout- ing  for  software-deﬁned  satellite  networks,  IEEE Trans.  Commun.  73  (1)  (2025) 454–468.

[4]  X. Zhu, C. Jiang, Integrated satellite-terrestrial networks toward 6G: architectures, applications, and challenges, IEEE Internet Things J. 9 (1) (2022) 437–461.

[5]  J. Liu, Y. Shi, Z.M. Fadlullah, N. Kato, Space-air-ground integrated network: a survey, IEEE Commun. Surv. Tutor. 20 (4) (2018) 2714–2741.

[6]  Bureau of Meteorology, Satellite communications and space weather, https://www. sws.bom.gov.au/Educational/1/3/2, 2024. (Accessed 1 March 2024).

[7]  X. Lin, B. Hofström, Y.-P. Eric Wang, G. Masini, H.-L. Maattanen, H. Rydén, J. Sedin, M. Stattin, O. Liberg, S. Euler, S. Muruganathan, S.-E. Löwenmark, T. Khan, 5G new radio evolution meets satellite communications: opportunities, challenges, and solu- tions, in: X. Lin, N. Lee (Eds.), 5G and Beyond: Fundamentals and Standards, Springer International Publishing, Santa Clara, 2021, pp. 517–531.

[8]  J. Sun, C. Wu, S. Mumtaz, J. Tao, M. Cao, M. Wang, An eﬃcient privacy-aware split learning framework for satellite communications, IEEE J. Sel. Areas Commun. 42 (12) (2024) 3355–3365.

[9]  J. Guan, Q. Zhang, I. Murturi, P.K. Donta, S. Dustdar, S. Wang, Collaborative in- ference  in  DNN-based  satellite  systems  with  dynamic  task  streams,  in:  Proceed- ing  of the  2024  IEEE  International  Conference  on  Communications,  IEEE,  2024, pp. 3803–3808.

[10]  3GPPTSG RAN WG1 Meeting #92bis, R1-1805114: NR-NTN channel modeling – ﬂat fading criteria, https://www.3gpp.org/ftp/TSG_RAN/WG1_RL1/TSGR1_92b/Docs/ R1-1805114.zip, 2018. (Accessed 20 April 2018).

[11]  X. Ding, X. Yang, B. Feng, C.-Y.-D. Sim, S. Wu, K.L. Chung, A dual-polarized wide- band parabolic reﬂector antenna with high aperture eﬃciency and low sidelobe level for L/S-band satellite communications, IEEE Access 12 (2024) 174859–174870.

[12]  Z. Sun, X. Lv, X. Zhu, Z. Liang, Y. Yang, A dual-band circularly polarized antenna with shared aperture for satellite-assisted internet of things communications, IEEE Internet Things J. 12 (7) (2025) 8461–8469.

[13]  B. Hadﬁeld, Satellite-systems cost estimation, IEEE Trans. Commun. 22 (10) (1974) 1540–1547.

[14]  M. Harounabadi, T. Heyn, Toward integration of 6G-NTN to terrestrial mobile net- works: research and standardization aspects, IEEE Wirel. Commun. 30 (6) (2023) 20–26.

[15]  X. Zhang, Y. Wang, X. Qin, Z. Zhang, H. Zhou, X. Shen, Link-level performance analy- sis of DVB standards in ultra-dense LEO satellite-terrestrial networks, in: Proceeding of the 2024 IEEE 99th Vehicular Technology Conference, IEEE, 2024, pp. 1–5.

[16]  Y. Liu, R. Zhang, Y. Zhou, Challenges and status of 3GPP non-terrestrial networks for mobile terminal testing, in: Proceeding of the 2024 International Conference on Microwave and Millimeter Wave Technology, IEEE, 2024, pp. 1–3.

[17]  3GPP TR RAN meeting #103, RP-240060: summary for NR NTN (Non-Terrestrial Networks)   enhancements,  https://www3.gpp.org/ftp/TSG_RAN/TSG_RAN/TSGR_ 103/Docs/RP-240060.zip, 2024. (Accessed 21 March 2024).

[18]  3GPP TSG RAN WG1 Meeting #109e, R1-2203159: discussion on coverage enhance- ment for NR NTN, https://www.3gpp.org/ftp/TSG_RAN/WG1_RL1/TSGR1_109-e/ Docs/R1-2203159.zip, 2022. (Accessed 20 May 2022).

[19]  3GPP  TR  RAN  meeting  #102,  RP-234078:  new  WID:  Non-Terrestrial  Networks (NTN) for NR phase 3, https://www.3gpp.org/ftp/TSG_RAN/TSG_RAN/TSGR_102/ Docs/RP-234078.zip, 2023. (Accessed 15 December 2023).

[20]  3GPP  TSG  RAN  WG1  Meeting  #116,  R1-2400132:  discussion  on  downlink  cov- erage enhancements for NR NTN, https://www.3gpp.org/ftp/TSG_RAN/WG1_RL1/ TSGR1_116/Docs/R1-2400132.zip, 2024. (Accessed 1 March 2024).

[21]  S. Wang, Y. Li, Q. Wang, M. Su, W. Zhou, Dynamic downlink resource allocation based on imperfect estimation in LEO-HAP cognitive system, in: Proceeding of the 2019 11th International Conference on Wireless Communications and Signal Pro- cessing, IEEE, 2019, pp. 1–6.

[22]  W. Gao, L. Wang, L.Z. Qu, Research on joint resource allocation for multibeam satel- lite based on metaheuristic algorithms, Entropy 24 (11) (2022) 1536.

[23]  H. Meng, N. Xin, H. Qin, D. Zhao, A recursive DRL-based resource allocation method for multibeam  satellite  communication systems,  Chin. J. Electron.  33  (5)  (2024) 1286–1295.

[24]  S. Guo, K. Han, W. Gong, L. Li, T. Feng, X. Jiang, An eﬃcient multi-dimensional resource allocation mechanism for beam-hopping in LEO satellite network, Sensors 22 (23) (2022) 9304.

[25]  M. Zhang, X. Yang, Z. Bu, Beam-hopping-based resource allocation in integrated satellite-terrestrial networks, Sensors 24 (14) (2024) 4699.

[26]  X. Li, L. Zhu, K. Chu, W. Shi, High-throughput satellite resource allocation strategy based on OFDM, in: Proceeding of the 2023 International Symposium on Networks, Computers and Communications, IEEE, 2023, pp. 1–6.

[27]  C. Lu, J. Shi, B. Li, X. Chen, Dynamic resource allocation for low earth orbit satellite networks, Phys. Commun. 67 (2024) 102498.

[28]  X. Zhong, H. Yin, Y. He, Y. Huang, Joint downlink power and time-slot allocation for distributed satellite cluster network based on Pareto optimization, IEEE Access 5 (2017) 25081–25096.

[29]  S.  Abdu,  S.  Kisseleﬀ,  E.  Lagunas,  S.  Chatzinotas,  Flexible  resource  optimization for GEO multibeam satellite communication system, IEEE Trans. Wirel. Commun. 20 (12) (2021) 7888–7902.

1760

[30]  T.S. Abdu, S. Kisseleﬀ, E. Lagunas, S. Chatzinotas, Power and bandwidth minimiza- tion for demand-aware GEO satellite systems, in: Proceeding of the 2021 IEEE Global Communications Conference, IEEE, 2021, pp. 1–6.

[31]  X. Chang, C. Wang, M. Pang, J. Wang, W. Wang, Resource allocation using deep re- inforcement learning in GEO multibeam satellite system, in: Proceeding of the 2023 IEEE International Symposium on Broadband Multimedia Systems and Broadcasting, IEEE, 2023, pp. 1–6.

[32]  I.  Leyva-Mayorga,  V.  Gala,  F.  Chiariotti,  P.  Popovski,  Continent-wide  eﬃcient and fair downlink resource allocation in LEO satellite constellations, in: Proceed- ing  of the  2023  IEEE  International  Conference  on  Communications,  IEEE,  2023, pp. 6689–6694.

[33]  S.-F. Hika, L. Zhu, T. Ji, H. Du, M. Chen, J. Liu, Joint allocation of downlink power, bandwidth and timeslots based on maximizing energy eﬃciency for GEO satellite communication systems, in: Proceeding of the 2024 IEEE 99th Vehicular Technology Conference, IEEE, 2024, pp. 1–6.

[34]  X. Zhong, Y. He, H. Yin, J. Wang, Z. Du, Joint power and timeslot allocation based on  delay priority  for multi-beam  satellite  downlinks,  in: Proceeding  of the  2017 International Conference on Progress in Informatics and Computing, IEEE, 2017, pp. 389–393.

[35]  3GPP, TR 23.501: system architecture for the 5G system (5GS), https://www.3gpp. org/ftp/Specs/2025-03/Rel-18/23_series/23501-i90.zip, 2025.  (Accessed  1 March 2025).

[36]  M.K. Arti,  Channel  estimation  and  detection  in  satellite  communication  systems, IEEE Trans. Veh. Technol. 65 (12) (2016) 10173–10179.

[37]  N. Letzepis, A.J. Grant, Capacity of the multiple spot beam satellite channel with Rician fading, IEEE Trans. Inf. Theory 54 (11) (2008) 5210–5222.

[38]  ITU, ITU-R P.618-13: propagation data and prediction methods required for the de- sign of earth-space telecommunication systems, https://www.itu.int/rec/R-REC-P. 618-13-201712-S/en, 2017. (Accessed 4 December 2017).

[39]  D.-H. Jung, H. Nam, J. Choi, D.J. Love, Modeling and analysis of GEO satellite net- works, IEEE Trans. Wirel. Commun. 23 (11) (2024) 16757–16770.

[40]  P.-S. Malik, R.-P. Singh, Y. Singh, N. Mittal, Parametric survey on cross-layer design approach for wireless sensor networks, in: Proceeding of the AIP Conference, AIP, 2023, p. 020017.

[41]  D.-K. Sah, D.-P. Kumar, C. Shivalingagowda, P.V.Y. Jayasree, 5G applications and architectures, in: D.N.-K. Jayakody, K. Srinivasan, V. Sharma (Eds.), 5G Enabled Se- cure Wireless Networks, Springer International Publishing, Tomsk, 2019, pp. 45–68.

[42]  Z. Zhao, Y. Cai, W. Hong, J. Jiang, M. Peng, T.Q.S. Quek, On the design of backscatter communications with retransmissions, IEEE Wirel. Commun. Lett. 11 (12) (2022) 2555–25592.

1761